Railpican P1A errata

- The CANBUS controller “u1 - MCP2515-I/P” interrupt is not connected to the raspberry pi interrupt pin. A patch-wire is required between pinhead connector pin 22 and u1 pin 12.

- The voltage regulator to feed the Raspberry pi from the 12V CANBUS terminal screw connector is becoming way to hot. Don’t connect the 12 V terminal, and feed the Raspberry pi separately from USB or similar.

Interrupt patch needed

Don’t connect – use separate Raspi power feed

Figure 1: Needed patches

Figure 2: Real life patch – but the yellow patch wire broke away from u1 pin 12

Figure 3: Gnd, CanL & CanH (but not 12V) being patched into the almost non existent DIN connector SKU of Merklin control head end. I have not found a single supplier for this connector.